Release note

2025.9.10 FW: 201301

- Changed the internal IP range of cameras

- When using CCTV mode with WAP, the default IP address was set to the A-class (10.x.x.x), and cameras were assigned IPs in the 192.168.254.x range. This caused video loss on NVRs connected to different subnets. To resolve this, the internal camera IP addresses have been changed to the B-class range (172.16.x.x ~ 172.31.x.x).

- Applied a patch to fix a boot issue caused by UBIFS error (ecc fail), which previously prevented the system from booting properly.

- Fixed an issue where the camera would reboot unexpectedly during reconnection.

- Fixed an issue where privacy mask coordinates were displayed incorrectly on 5-megapixel cameras.

- Fixed an issue where the time shown in the AI event notification email content did not match the email's received timestamp.

- Improved cyber security

- Improved AI Bridge & Pro Camera compatibility

- Improved HDD recovery process

2024.2.21 FW: 201025

- Improve Web resource.

- Performance improvements & minor Bug fixes.

- Improve Nand memory compatibility.

*please note that It is not possible to downgrade FW to the previous version.

xxxxx.1.1xxxxx.32 → xxxxx.1.2xxxxx.32 OK / xxxxx.1.2xxxxx.32 → xxxxx.1.1xxxxx.32 NG

2023.6.8 FW:101014

- Performance improvements & minor Bug fixes

- Enhanced security

- Updated network chip compatibility (8/16CH)

- Removed menu regarding to FR, LPR database to ensure system stability.

- Support for Non-ActiveX browsers